RPCR.DOC     MVS Box850 Merrimack,NH     5/6/92

The Remote Power Card Reset version (RPCR) is a 1/4 slot card that

plugs into an XT/AT type computer.  This card reboots the computer

should it become locked up for hardware or software reasons.

The RPCR functions by maintaining a timer seperate from the computer.

When this timer reaches terminal count it causes the same effect as

pressing the Reset button.  Reboot can be prevented by writing a 1 and

then a 0 to the Counter Clear Address.  This causes the count to begin

again.  The computer will only be reset if this routine does not run,

such as when the computer is locked up.

The card installs in one of the motherboard slots.  Since the PC Bus

has no polarizing keys it is very important that the component side of

the card be facing the same direction as the other adapter cards.

*************************************************************

*   WARNING !!!  THE RPCR MUST FACE THE SAME DIRECTION AS   *

*      OTHER CARDS IN THE PC OR DAMAGE WILL RESULT !!!!     *

*   PLUGGING THE RPCR IN BACKWARDS WILL VOID THE WARRANTY.  *

*************************************************************

The only connection to the card other than the PC Bus is a thin wire

with a connector on the end.  This wire plugs onto one of the

motherboard pins that go to the Reset button.  You can either remove

the cable that goes to the Reset button or splice into it.  If the RPCR

does not function then try the other one in the pair.  The RPCR signal

is an open-collector transistor so relays, indicators, and other

devices can be activated instead of Reset if necessary.

Testing is simple.  With no special software running and the RPCR

installed the computer will reboot once every four minutes.  In each

cycle there is a two minute delay followed by Reset being activated for

two minutes more.  Most computers boot faster than the Reset Delay

period giving AUTOEXEC.BAT enough time to run the user program.  During

normal operation the user program clears the timer at intervals of less

than two minutes to keep from rebooting.

Reset remains asserted for two minutes.  This gives modems, phonelines,

printers and other peripherals time to settle into a known initial

state.  When released the computer starts booting immediately.

If necessary the Reset Delay time can be altered and the Counter Clear

Address may be changed via jumpers.  Reset Delay values are guaranteed

to be equal to or greater than indicated.  Remember to cut the default

run before adding new jumpers.

Reset Delay Jumpers:     Settings:            22-16 = 4 sec

J9 10 11 12 13 14 15    J22-J14 = 1/16 sec  22-18 = 8 sec

o  o  o  o  o  o  o      22-15 = 1/8 sec   22-12 = 16 sec

o  o  o  o  o  o  o      22-21 = 1/4 sec   22-13 = 32 sec

J16 17 18 19 20 21 22      22-20 = 1/2 sec   22-10 = 64 sec

22-19 = 1 sec     22-17 = 128 sec (default)

22-11 = 2 sec     22-9  = 256 sec

Counter Clear Address:

J1 J2 J3 J4              121h = J1-J6 2-5 3-7 4-8

o  o  o  o              181h =   1-5 2-6 3-7 4-8 (default)

o  o  o  o              221h =   1-6 2-5 3-8 4-7

J5 J6 J7 J8              281h =   1-5 2-6 3-8 4-7

Several demo programs are included showing how to use the RPCR with

Assembly, BASIC, and C.  These programs intentionally lock up the

computer when a key is pressed.  The RPCR will reboot it!

WARRANTY

MVS will repair free of charge boards suffering from manufacturing

defects for a period of 60 months after shipping.  This will be done

only if it has been determined by MVS that the product has not been

misused (improper supply voltages, physical alteration, etc.).

Otherwise a minimum service charge is required.  In either case it is

necessary to contact the company for a Returned Merchandise

Authorization (RMA) number before shipping the unit.

MVS assumes no responsibility for damage or other loss resulting from

use of this product or modifications associated with it.

No part of the RPC circuit shall be duplicated in any way without

permission from MVS.  The design is protected under Civil and Criminal

Law.  Manufacturing licenses for products are available from MVS in

addition to significant OEM discounts.  Contact the company for more

information.