---------

*INSTALL*

---------

To install the NovaBack program, simply insert the distribution

disk in floppy drive A or B, then type A:INSTALL (or B:INSTALL) at

the DOS prompt, then press ENTER. You may also attach to the floppy

drive first, then just type INSTALL at the DOS prompt, then press

ENTER.

The INSTALL program has been included to install the NovaBack

program files. It also offers optional installation of the automatic

backup scheduler, which will result in modification of your

AUTOEXEC.BAT and CONFIG.SYS files on your BOOT drive.

Be sure the proper drivers are called out in your CONFIG.SYS file.

These drivers (with the exception of IBM brand system and those

installing Qic 36 type tape drives) are supplied by the manufacturer

and required to be ASPI compatible.  If you are in doubt as to

whether your SCSI controller is ASPI compliant, you should contact

the manufacture for further information. Also, you should verify

that the driver you are using is current.

Manually adding the Scheduler

-----------------------------

The driver 'NSD.SYS' is required in the CONFIG.SYS to use the

scheduling feature in NovaBack, which allows for automatic backup

at a pre-designated time of day. The  form of the following line

must also appear in the AUTOEXEC.BAT:

COPY <source_path>\BACKUP.SCD NOVASCHD

Option: /F  :For Lantastic Users (Will beep if not included)

An Example is "COPY C:\NOVABK\BACKUP.SCD NOVASCHD".  BACKUP.SCD is

a schedule data file that is created when you create your schedule.

NOVASCHD is a memory location where schedule data must be copied.

Note: The scheduler will launch a job by executing a command at a

DOS prompt.  For backup, we provide two utilities, BKP.EXE (Backup)

and BKVER.EXE (Backup & Verify).  You may type their names by

themselves from the Novaback directory to get a list of parameters.

Examples of their use are: "BKP {SRDHY} C:"  or "BKVER {G2V} {G2V}

(ProcName.BUP)".  The *.BUP files are created with Novaback during

the Select files for Backup.  RST (Restore), TAPEU (Utilities), &

BVER (Verify) may also be used.

---------------------------

*BVER command line options*

---------------------------

Although you may enter the commands BKP, RST, and BKVER with no options

in order to obtain immediate help, such is not true for BVER.

Therefore this information explains options for the BVER command.

Note that simply running BVER alone will cause a verify of dataset 1

to begin, by default.

Command line parameter format is [)LOGNAME.TXT] [{options}] [drives]].

Options must precede files to which they apply.   Options must be enclosed in

braces and may be separated by commas but may NOT contain blanks.

Batch files may contain options, files to backup or other batch file names.

Both the log name and drives options are optional.

Options are:

R    Rewind before starting (default).

Fn   Verify from the n'th backup set on the tape.

Gn   Set logging level. n = 0, 1, or 2.

I=(drive-name)  Specify tape unit name(s). Multiple entries for cascading.

K=(n,n,..)  Specify random changer sequence.

Z    Verify the last backup set written on the tape.

#n   Limit the number of tape buffers used to 'n' (16k) buffers.

A minus (-) ahead of an option resets it to false.

Example:  C>bver )VLOG.TXT {z} c:  Verifies back of drive C, logs to VLOG.TXT

--------

*TSETUP*

--------

Novaback is designed to automatically sense the make and model of

tape drive(s) that is/are connected to your computer system and

properly configure itself accordingly.  However, there are some

occasions where you may need to customize the way that Novaback

communicates with a tape drive.  Specifically, you might have a

tape drive that supports more than one density, or a tape drive

that provides data compression capabilities.  Because Novaback is

self configuring, you are not normally able to alter the density or

compression mode of your drive.  This limitation can furnish a

major barrier when interchanging data from one kind of hardware to

another.

Almost all of the newer model tape drives support multiple

densities.  All of them automatically switch to the correct density

upon tape insertion. Foe example, the Exabyte model 8505 tape drive

has the ability to store 10 gigabytes on a 112 meter tape.  It also

has compatibility with four different densities in order to be

compatible with all of the older model 8mm drives.

The EXB-8500 and EXB-8505 drive cannot automatically switch modes

for you when you are creating a new tape.  It can only use its

default setting (which is usually set for the highest density by

the factory).  This means that an EXB-8500 normally generates a 5

gigabyte tape, and the EXB-8505 generates a 10 gigabyte tape.

These tapes can't be read by the older EXB-8200.  As a matter of

fact, if you try it, the EXB-8200 will respond with an error

claiming that a defective tape was inserted.

When you use Novaback to generate tapes that are planned for

transport to another system, you need to investigate the type of

equipment that is being used at the destination site.  Their

equipment must be able to read tapes created on your tape drive.

If they are using older hardware, they may encounter a problem

reading your data because the tape is too sophisticated for their

equipment.  The solution is to force your higher capacity tape

drive to run at a lower density than its maximum.  This change in

density requires a special effort on the software's part.  In order

for Novaback to generate an 8200 density tape in an EXB-8500 or

EXB-8505 drive, Novaback must first select a different mode of

operation.  Novaback normally generates tapes in the drives default

density unless specifically told to do otherwise...via the TSETUP

program.

The following sections describe the process for using the TSETUP

utility to customize the way that Novaback communicates with your

tape drive.  By changing the tape drive's density setting within

TSETUP, you can control the density of output tapes and maintain

compatibility with the destination's older hardware.

To begin the TSETUP program, switch to the drive and subdirectory

where you installed the Novaback program and type:

TSETUP

Upon starting, the TSETUP program reads a file in from the disk

called DRIVES.INI.  This data file contains specific information

about your computer system's tape drives.  If this is the first

time that TSETUP is run, the DRIVES.INI file doesn't exist yet.

TSETUP displays a list of configured tape devices on the screen.

This screen allows you to add, configure, and remove tape

configurations for the Novaback program.

Adding a New Drive Configuration

--------------------------------

To add a new tape drive configuration, press the F2 key.  Novaback

immediately scans your system hardware and displays a list of all

the tape devices that it finds.  In order for TSETUP to recognize

a tape drive, it must be turned on and connected to the system.

The list displays the tape drive's unit address, its make and

model, and a description.  If you have more than one tape device,

use the UP, DOWN, PAGE UP and PAGE DOWN keys to move the highlight

bar to the desired selection and then press ENTER.

Modifying a Device Configuration

--------------------------------

The device configuration screen allows you to customize the

different tape drive settings for each specific hardware device.

The screen gives you the option of changing the device name,

description, block type and density.

Name

----

The name field is required throughout the program to identify

the tape configuration.  Each device that you configure in TSETUP

must have a unique name.  Its a good idea to keep the name short

whenever possible.

Description

-----------

Enter a description of the tape device configuration here.  The

description field is optional.  Its contents are not used anywhere

in the program.  This field is commonly used to provide any

specific information about the tape drive configuration that

doesn't belong in the name.

Drive Density

-------------

The drive density field is used to specify a density.  On tape

drives that have multiple densities, you can press ENTER and cycle

through the different options.  For example, to change an Exabyte

8505 drive and make it write tapes in an EXB-8200  compatible

density, press F2 until 8200 is displayed here.  If you have a tape

drive that has built in hardware data compression, use this field

to toggle the compression mode on and off.

For drives that support multiple densities, such as the Exabyte

8505, a quarter inch drive, or a 4mm DAT with compression,  it's

recommended that you add a different configuration for each

density.  Each configuration must have a unique name.  Adding a

separate configuration for each density allows you to easily select

the desired density from within the Novaback program.  Novaback

will display a list of all the configuration names and allow you to

pick the density of your choice.

Saving Your Custom Tape Configuration

-------------------------------------

When you are satisfied with the tape drive setting, press F10 to

save and return to the initial TSETUP screen.  Press ESCAPE if you

decide to cancel the changes and return back to the initial screen

without saving the changes.

Application

-----------

Once you have made what ever changes you deem necessary, run

Novaback and push "F4" from the main menu.  Highlight the currently

installed drive with the cursor using the arrow keys.  Push the

"Enter" key and your alternate drive will appear in the highlighted

area. If you see the same drive entry, no changes may be necessary.

Push "F10".

-------

*TAPEU*

-------

This is a command line program to rewind, format, or erase a tape.

The parameters are processed by the program in the order in which

they are given.  This allows parameters to be setup to perform a

sequence of functions on different tapes if desired.  These

parameters  may be submitted in a batch file and (if you really

care to) a batch file may invoke other batch files.

The program will return the following codes indicating the

success of the requested functions.

0       All functions completed ok

1       No tape unit found

2       Error initializing tape unit

3       Tape cannot be formatted

4       Error formatting DAT tape

5       Error formatting QIC tape

6       Erase tape error

7       No command line parameters

8       Batch file not found

9       Invalid parameter

The valid parameters are:

/U:n  Set tape unit address to be accessed to 'n'

/F    Format a QIC tape or format a DAT tape to default

partition size (16m)

/F:n  Format a DAT tape with partition 1 = 'n' megabytes

/E    Erase a tape (erased to end of tape)

/L    List available devices

/S    Set silent running.  No messages are sent to stdout.

/R    Rewind tape

/J    Rewind and Eject Tape

/B:file_name  Get parameters from a batch file

Note that erase end of tape is NOT automatically proceeded by a

rewind.  To be sure that the entire tape is erased, precede the /E

with a /R.  This allows erase to be used from a known position on

the tape without rewind, if desired.

Run TAPEU with no parameters to get a summary of valid parameters

and return codes.

If silent running is not specified, error messages will be sent to

stdout.  If no errors occur, no messages will be output.

------------

*IBM DRIVER*

------------

IBMASPI.SYS - Device driver for IBM host adapters

May be loaded as:

DEVICE=<source_path>\IBMASPI.SYS

For example: DEVICE=C:\NOVABK\IBMASPI.SYS

One or more of the following options may be required depending on

the hardware conditions and model of IBM.

/N     = Specifies Native Mode (Imbedded Controllers).

/Pxxxx = Specifies the Port.

--------

*QIC 36*

--------

The ONLY host adapters that are supported are Everex and Compex.

May be loaded as:

DEVICE=<source_path>\ASPI36.SYS /P130 /D1 /I3

For example: DEVICE=C:\NOVABK\ASPI36.SYS /P130 /D1 /I3

where

/Pxxx = Specifies the port address of the card.

/D1   = Specifies the DMA.  DMA channel one is currently the

only supported option.

/Ix   = Specifies the interrupt.

-----------------

*Troubleshooting*

-----------------

This portion of the document outlines procedures for quickly and

accurately troubleshooting problems occurring with Novaback and a

SCSI tape drive.  It outlines the basics for troubleshooting system

conflicts and resolving issues ranging from   "No tape unit found"

all the way to full system lockups.  Of course success isn't always

guaranteed, but over 90% of non-instructional problems (user

errors) can be solved by following this guide.  Even when you get

a weird problem, or something that you know doesn't make sense, use

this guide as a starting point, so you can eliminate the basic

stuff and get it out of the way.  Also, in the event of a serious

problem you can cruise through these steps to quickly determine

whether or not there is a hardware problem.

SCSI ID

-------

Check the SCSI ID on the tape drive.  As a general rule,

stay away from SCSI ID 0, 1, or 7 for a tape drive.  In the event

of a problem, you should change the SCSI ID if it its on 0, 1, or

7 . . . even if it was working before!  Why?  These SCSI id's are

reserved.  SCSI id's 0 and 1 are usually hard disks, and 7 is the

id of the host adapter.

SCSI TERMINATION

----------------

This can be tricky so do not confuse termination with term power.

This is another step that shouldn't be skipped . . . even if

everything was working fine before! What is termination?

Termination is the installation of

either resistor packs, external plugs, or DIP switches on SCSI

devices and host adapter cards to eliminate bus impedance

mismatches and improve data transmission reliability.  Most host

adapter cards are terminated with 3 resistor packs, which measure

about 1 1/4 inches long, 3/8 inch high, and 1/8 inch thick.  These

are inserted directly into the printed circuit card.  All internal

SCSI devices have terminating resistors installed in them when

shipped.  External SCSI devices are generally shipped without

terminating resistors.

So what does all this mean?  Well proper termination means that

there are only two devices on the SCSI chain are terminated, the

first device on the bus and the last one.

Note:  External SCSI drives are just internal drives that are

installed into a shoebox case.  If the drive is removed from the

case, you can look at the termination on it.  Whoever assembled

the case should have removed the termination from the internal

drive. Its common to see situations where this doesn't  happen.

Its good practice to check the internal termination on external

drives whenever problems occur.

Example A:

You have a single SCSI card an internal tape drive (and no other

SCSI devices attached to the card).  Both of them should be

terminated (remember that SCSI cards are also SCSI devices; hence

the name host adapter).

Example B:

You have an internal SCSI card, internal SCSI hard drive, internal

SCSI tape drive, and internal SCSI CD-ROM drive. The termination

must be removed from two of these devices.  Which ones?  Well, you

must remove it from the devices in the middle of the chain.  Look

at the SCSI ribbon cable in the computer.  The ribbon comes off the

controller and goes to the first device in the SCSI chain, (hard

drive), then to the next device on the chain (tape drive) and

finally to the last device in the chain (CD-ROM).  In this example,

the hard drive and tape drive are in the middle of the chain, so

they need to have their termination removed.

Example C:

You have an internal SCSI card, internal SCSI hard drive and

external SCSI tape drive.  Remove termination from the middle

device on the chain, which in this case is the SCSI card.  Whenever

you have both internal and external devices attached to your SCSI

card, you always remove the termination from the SCSI card and then

follow examples A and B. If you remember, we stated earlier that

external tape drives do not have termination.  In the above example

C, we need to add termination to the external tape drive.  This can

be done two different ways.  One is to take the internal drive out

of the case and plug in terminators right into its circuit board

(not recommended).  The other way is to plug in an external

terminator on the back of the case.  Most external cases have two

SCSI connectors in the back.  One of the connectors has a cable

going to the host adapter, and the other one should have a

terminator attached to it.

If you have a SCSI device that only has one connector on the back,

you can install a pass-through terminator.  This is a special

looking terminator that sandwiches itself between the SCSI drive

and the SCSI cable.

Drivers

-------

Make sure that the correct aspi drivers are being loaded in the

config.sys.  If a customer is using an Adaptec card, have them

eliminate AHASHELL.SYS.  Switch them over to Adaptec's aspi

manager. AHASHELL is no longer supported.

Use this list as a driver guide:

SCSI HOST ADAPTER               DRIVER

Adaptec 151X                    ASPI2DOS.SYS

Adaptec 152X

Adaptec 154X                    ASPI4DOS.SYS

Adaptec 164X                    ASPI4DOS.SYS

Adaptec 174X                    ASPIEDOS.SYS

**Use ASPI4DOS.SYS when card is in standard mode.

Adaptec 274X                    ASPI7DOS.SYS

Always IN-2000                  ASPIDRVR.SYS

Always AL-7000                  AL7ASPI.SYS

AMI 9000X                       ASPIKERN.SYS (up to 2 devices) or

ASPIHDD.SYS (more than 2 devices)

Bus Logic                       BTDOSM.SYS

DTC 3280                        ASPI3X80.SYS

DTC 3290                        ASPI3X90.SYS

DPT 2011/XX                     DPTDDL.SYS

DPT 2012/XX

Future Domain MCS-7XX           ASPIFCAM.SYS

Future Domain TMC-8XX

Future Domain TMC-16XX

Future Domain TMC-17XX

IBM                             IBMASPI.SYS (provided by Novastor)

UltraStor 14F                   USPI14.SYS

UltraStor 24F                   USPI24.SYS

UltraStor 34F                   USPI34F.SYS

Vanilla Boot

------------

This is a great step for distinguishing between hardware problems

or software problems.  Basically, you want to boot the system into

a vanilla environment where no other software is being loaded.

Eliminate everything from the CONFIG.SYS except for the following:

DEVICE=ASPI4DOS.SYS (or other aspi driver).

DEVICE=HIMEM.SYS

DOS=HIGH

FILES=50

Also, rename the AUTOEXEC.BAT to AUTOEXEC.BAX so the system

doesn't run it when it boots.  If this fixes the problem then

instruct the customer to add lines back into the config.sys and

autoexec.bat one at a time so they can pinpoint which line is the

trouble maker.  If this step doesn't solve the problem, then we

know that there is some type of hardware problem.

Hardware Problems

-----------------

At, this point, if none of the above have solve your problem, we

can now conclude that there is a hardware problem.  Here's the

common things to look for in the hardware:

Make sure that the drive supports the tape being used.

Try using a different tape. Try cleaning the tape drive with a

cleaning cartridge. Make sure the terminators are not plugged in

backwards. If an external device is the last device in the chain,

make sure that it is powered on (in order for the terminators to be

effective). Try turning off sync negotiation on the SCSI host

adapter card. Try turning off parity on the SCSI host adapter card.

(Be careful with this one, it can make things worse). Try turning

off disconnect on the SCSI host adapter card. Try slowing down the

transfer speed on the SCSI host adapter to the lowest setting.

Make sure only 1 device (including the adapter) is supplying

termination power. Remove all but 1 device from the SCSI chain.

Minimize cable length (1 foot). Verify that each SCSI device on the

chain is set to a unique SCSI ID. Check with the manufacturer of

the tape drive for current firmware.

You also should have FILES and BUFFERS set to at least 20.

If you have any problems using NovaBack, our technical support

staff is available to assist you at (818) 879-4878.  Please have

your manual and orignal disk handy.