1.System Resources of The PCnet Chipset

A. LANCE(AM7990)

address     : 24 bits

I/O address : 300h, 320h, 340h & 360h(jumpper select)

ROM address : C8000h, CC000h, D0000h & D4000h

(subject to I/O addr)

DMA channel : 3, 5, 6 & 7(DRQ/DACK#, jumpper selected)

(cascade DMA & request bus from AT-bus)

IRQ request : 3, 4, 5 & 9(IRQ, jumpper selected)

Chip ID(CSR): N/A

Registers   : CSR 0 - 3(16 bit format)

Init block  : 24 bits format

LED         : LED0, LED1, LED2, LED3

B. PCnet-ISA(AM79C960)

address     : 24 bits

I/O address : 300h, 320h, 340h & 360h(jumpper select)

ROM address : C8000h, CC000h, D0000h & D4000h

(subject to I/O addr)

DMA channel : 3, 5, 6 & 7(DRQ/DACK#, jumpper selected)

(cascade DMA & request bus from AT-bus)

IRQ request : 3, 4, 5, 9, 10, 11, 12, 15(IRQ, jumpper selected)

Chip ID(CSR): 88,89([27:12]=0003h, [11:1]=00000000001)

Registers   : CSR 0 - 126, ISACSR 0 - 7(16 bit format)

Init block  : 24 bits format

LED         : LED0, LED1, LED2, LED3

C. (1). PCnet-VL(HILANCE)

address     : 32/24 bits

I/O address : 200h - 3FFh(per 20h offset)

ROM address : (subject to system/peripheral card design)

DMA channel : LREQ#, LGNT# from local bus(VL sepc.)

IRQ request : As per offset 8 of I/O space

(Hilance IRQ Register)

Chip ID(CSR): 88,89([27:12]=2430h, [11:1]=00000000001)

Registers   : CSR 0 - 126, BCR 0 - 21(32/16 bit format)

Init block  : 32/24 bits format

LED         : LED0, LED1, LED2, LED3

(2). PCnet-PCI(PERSEDIO)

address     : 32/24 bits

I/O address : 0000h - FFFFh(determined by PCI BIOS)

ROM address : (subject to system/peripheral card design)

DMA channel : REQ#, GNT# from local bus(PCI sepc.)

IRQ request : 0 - 15(INTA#, determined by PCI BIOS)

Vendor ID   : 1022h(function 0)

Device ID   : 2000h

Chip ID(CSR): 88,89([27:12]=000xh, [11:1]=00000000001)

Function    : 0

Registers   : CSR 0 - 126, BCR 0 - 11(32/16 bit format)

Init block  : 32/24 bits

LED         : LED0, LED1

(3). PCnet-PCI & SCSI-PCI(GOLDEN-GATE)

address     : 32/24 bits

I/O address : 0000h - FFFFh(determined by PCI BIOS)

ROM address : (subject to system/peripheral card design)

DMA channel : REQ#, GNT# from local bus(PCI sepc.)

IRQ request : 0 - 15(INTA#/INTB#, determined by PCI BIOS)

Vendor ID   : 1022h(function 0/1)

Device ID   : 2000h(function 1)

Chip ID(CSR): 88,89

Function    : 1

Registers   : CSR 0 - 126, BCR 0 - 11(32/16 bit format)

Init block  : 32/24 bits

LED         : LED0, LED1

D. PCnet-Jumpless(LAN SHARK)

address     : 24 bits

I/O address : 200h - 3FFh(per 20h offset)

ROM address : C0000h - DE000h

(per 2000h offset, subject to size)

DMA channel : 3(DRQ/DACK3#), 5(DRQ/DACK5), 6(DRQ/DACK6) &

7(DRQ/DACK7). (0 - 7, P&P sepc.)

(cascade DMA & request bus from AT-bus)

IRQ request : 3(IRQ3), 4(IRQ4), 5(IRQ5), 9(IRQ9), 10(IRQ10),

11(IRQ11), 12(IRQ12) & 15(IRQ15)

(1 - 15, P&P sepc.)

Chip ID(CSR): 88,89

Registers   : CSR 0 - 126, ISACSR 0 - 8(16 bit format)

Init block  : 24 bits format

LED         : LED0, LED1, LED2, LED3

E. First 16 Bytes in I/O Space(EEPROM shadow)

base of I/O space is hardwired, system configureable or

system relocateable.

BASE +  00h - 05h Ethernet/802.3 MAC address

06h - 07h Reserved(must be zero)

08h	  0000xxxxb where xxxx=IRQ number for Hilance

09h       Hardware Identification

0Ah - 0Bh Vendor specific

0Ch - 0Dh Checksum of bytes 00h - 0Bh & 0Eh - 0Fh **

0Eh - 0Fh Ascii character "WW"

Hardware Identification(09h)

00h or FFh PCnet-ISA/AM2100/AM1500T

01h        PCnet-ISA+(jumpless)

10h        Hilance(Always 4 INTR's)

11h        Persidio(PCnet-PCI)/Golden Gate

2. Detection Mechanism for Each Individual Adapter

A. PCnet-ISA/LANCE

(a.) scan/input I/O address     = 300h, 320h, 340h, 360h

(b.) check      "WW" at I/O address offset 0Eh & 0Fh

Checksum at I/O address offset 0Ch & 0Dh **

Hardware Identification offset 09h

(c.) get        IRQ             = 3, 5, 6 & 7 (input/default)

DMA             = 3, 4, 5 & 9 (input/default)

(d.) check      input with detected parameters conflict

(e.) device initialization

B. (1). PCnet-VL(HILANCE)

(a.) scan/input I/O address     = 200h - 3FFh(per 20h offset)

(b.) check      "WW" at I/O address offset 0Eh & 0Fh

Checksum at I/O address offset 0Ch & 0Dh **

Hardware Identification offset 09h

(c.) get        IRQ             = BCR21

(d.) check      input with detected parameters conflict

(e.) device initialization

(2). PCnet-PCI(PERSEDIO/GLODEN GATE)

(a.) use        mechanism 1 CF8h, CFCh(8:bus * 32:dev = 256)

mechanism 2 CF8h, CFAh(8:bus * 16:dev = 128)

to detect valid Vender ID

bus             = 0

devices         = 0

vender ID       = NOT (FFFFh)

(b.) scan       bus             = 0 - 7

devices         = 0 - 31(mech. 1)/15(mech. 2)

vender ID       = 1022h

device ID       = 2000h

function        = 0/1

(c.) set        base addr reg   = base I/O address(10h)

(d.) check      "WW" at I/O address offset 0Eh & 0Fh

Checksum at I/O address offset 0Ch & 0Dh **

(e.) set        INT line        = IRQ number(3Ch)

(f.) device initialization

C. PCnet-Jumpless(LAN SHARK)

(a.) scan       I/O address     = 200h - 3FFh(per 20h offset)

(b.) check      "WW" at I/O address offset 0Eh & 0Fh

Checksum at I/O address offset 0Ch & 0Dh **

Hardware Identification offset 09h

(c.) get        IRQ             = ISACSR 8

DMA             = ISACSR 8

(d.) device initialization

** Checksum is a simple sum of bytes 00h - 0Bh & 0Eh and 0Fh. The checksum

result is stored as least significant byte first. For example, the least

significant byte of the first checksum is stored at 0Ch and the most

significant byte at 0Dh.

3. Scan for PCnet devices

IF (I/O address specified)

IF (not match with "WW" || checksum || hardware identification)

Error_Out;

ELSE

{

IF (BUS type specified)

scan specified bus and get proper I/O address;

ELSE

scan PCI/PnP/VL/ISA bus and get proper I/O address;

IF (not match with "WW" || checksum || device identification)

Error_Out;

}

get parameters DMA, IRQ;

IF (parameters conflict with user_provide_infomation if applicable)

Error_Out;

initialize adapter/ device driver accordingly.

4. New Keywords and their Explanation

(i) DMAROTATE

This keyword applies to PCnet-ISA and PCnet-ISA+ only.

When present, this keyword will force the system 8237 in rotating priority

mode. The default mode for 8237 is fixed priority.

(ii) BUSTYPE = {PCI/PNP/ISA/VESA}

This keyword applies to all PCnet family devices.

If this keyword is specified with one of the above mentioned options, the

driver software will only scan the specified bus for the presence of

PCnet family device. If this keyword is not specified the driver software

will scan all the busses.

(iii) TP

This keyword applies to all PCnet family devices.

When present, this keyword will force the PCnet-XXX device into using the

10BaseT port. The chip has to use the 10BaseT port even if no link beat

pulse is being generated by the 10BaseT hub.

(iv) LED0 = XX

Where XX = hexadecimal value for BCR5 register.

This keyword applies to all PCnet family devices.

When present, this keyword should have a hexadecimal value which indicates

the function of the led0. The driver software reads the hex value of the

keyword and programs BCR4 with the value specified. The user has to be

careful about the hex value because the hex value is programmed directly into

the chip register.

Some legal values which should be documented in our manual are:

0 - Collision activity on the network

2 - Indicates PCnet is jabbering on the network

4 - Indicates promiscous receive activity on the network

8 - Indicates the current receive polarity condition

10 - Indicates the transmit activity

20 - Indicates receive activity for this station

40 - Indicates the current link status

30 - Indicates the transmit and receive activity for this station

34 - Indicates the transmit and promiscous receive activity

Although, it is possible to have some more combinations, we feel that the

above list should be the only published combinations for the led keyword.

(v) LED1 = XX

Same as (iv). Program BCR5.

(vi) LED2 = XX

Same as (iv). Program BCR6.

(vii) LED3 = XX

Same as (iv). Program BCR7.