Last updated: 11/04/88

CARTOG

A Cartography Program from BYTE Magazine

This disk contains several versions of the program CARTOG.PAS

described  in  "Mapping the World in Pascal" by Robert Miller and

Francis Reddy (BYTE,  December 1987,  page 329). The program is a

simple tutorial which allows you to view the properties of various

map projections on a graphics screen.  The  code shows how many of

the more common projections are calculated,  but  is  neither  the

best nor the fastest means of doing so -- its purpose is the clear

illustration of how maps are computed.  To make  start-up  of  the

programs a little easier, we  have included batch files. See below.

Files on This Disk

CARTOG   DOC  This documentation

CARTOG   PAS  Source code from BYTE, December 1987

METAWNDO EXE  Auto-loading and terminating graphics driver

PRTSCRN  EXE  "                      "     screen dump

RUN      BAT  Use this if you have no math coprocessor

CARTOG   COM

RUN87    BAT  Use this if you have a math coprocessor

CARTOG87 COM

WORLD    DAT  6,000-point coastline database

If  you have no 80x87,  just type "RUN" at the DOS prompt; if

you do  have  an  80x87  installed,  type "RUN87".

Getting  Started

The  MetaWindow  graphics  driver   allows us to support CGA,

EGA,  AT&T,  and  Hercules-type graphics cards.   The driver will

first  test  for  the  card on your system,  then report the card

detected  above  the  CARTOG   menu.   You probably won't have to

worry  about  it,  but  if  you  are  using  a  multi-mode  card,

MetaWindow  may  not come to the correct conclusions!   Check the

"Graphics:  "  line above the Menu -- if it does not match one of

the  mode's  of your card,  or if CARTOG appears to run but draws

nothing on the graphics screen,  you may need to modify the batch

file  by  adding the appropriate command switch from those listed

below.

Switch Graphics Board                           Resolution

-----------------------------------------------------------------

/A:1  AT&T Graphics Adaptor,                    640x400 2-color

/A:2  AT&T Display Enhancement Board (DEB),     640x200 16-color

/A:3  AT&T Display Enhancement Board (DEB),     640x400 16-color

/C:1  IBM Color Graphics Adaptor (CGA)          640x200 2-color

/E:1  IBM Enhanced Graphics Adaptor (EGA)       640x350 mono

/E:2  IBM Enhanced Graphics Adaptor (EGA)       320x200 16-color

/E:3  IBM Enhanced Graphics Adaptor (EGA)       640x200 16-color

/E:4  IBM Enhanced Graphics Adaptor (EGA, 128K) 640x350 16-color

/E:5  IBM PS/2 Video Graphics Array (VGA)       640x480 16-color

/E:6  IBM Enhanced Graphics Adaptor (EGA)       640x350 2-color

/E:7  IBM PS/2 MultiColor Graphics Array (MCGA) 640x480 2-color

/H    Hercules/AST Monochrome Graphics Adaptor, 720x348 mono

/O    Toshiba 3100,                             640x400 mono

/T:1  Tecmar Graphics-Master Adaptor,           720x352 mono

/T:3  Tecmar Graphics-Master Adaptor,           640x200 16-color

/T:4  Tecmar Graphics-Master Adaptor,           640x400 16-color

/U:1  Video-7 Vega Deluxe,                      640x480 16-color

/V:1  Everex Edge Adaptor,                      640x200 16-color

/V:2  Everex Edge Adaptor,                      640x400 4-color

For example, the driver will identify the PS/2 video board as

"IBM EGA,  640 x 350" rather than "IBM VGA,  640 x 480." Altering

the  batch  file to read "Metawndo CARTOG /E:5" tells the program

what hardware to draw to. Please note that the MetaWindow driver,

although a terminate-and-stay-resident program, eliminates itself

from  memory when you exit CARTOG.   An additional note:  Several

users  have  reported  problems   with certain cards (notably the

Paradise  EGA  card)  and the Metawndo driver.   While we cannnot

solve  specific  hardawre  problems  (other  than  providing  the

command  line  switches  above),  we  would  appreciate   problem

reports  so  that we may pass them on to MetaGraphics,  makers of

the  driver.   Also,  we  are readying a version of CARTOG in the

latest  version  of  Turbo  Pascal,  which  includes    extensive

graphics support.

To make CARTOG more flexible,  you may specify a data file to

read on the command line.   Otherwise,  CARTOG looks for the file

WORLD.DAT.

T H E  M A P S

The following briefly  describes  the types  of maps CARTOG draws.

For  more information,  see  the  BYTE  article  and  consult  any

of the references mentioned therein.

Map 1: The Mercator

o  Preserves shape but distorts area, especially

in polar regions;

o  Poles lie at infinity;

o  Meridians are straight, equally spaced lines;

o  Parallels are straight lines spaced unequally, closest

near Equator;

o  Meridians and parallels intersect at right angles.

Quick.   Which  is  bigger:  Greenland  or South  America? If

you're  not at all sure,  part of the blame rests with the Merca-

tor  projection.   Greenland is only one-eighth the size of South

America,  but  Mercator's  projection  makes it into a full-blown

continent!   The  map  is   often   the   only    two-dimensional

representation   of  the  Earth  many students see.   Its primary

purpose  is  navigational.   A  ship  or plane following the same

compass  course  will  travel  a  straight line when plotted on a

Mercator  map.   Gerhardus  Mercator developed this projection in

1569  specifically  as  an   aid   to   navigation.    Mercator's

projection  is  classed as a cylindrical,  and at first glance it

looks  much   like   the   Equidistant  Cylindrical (Map 2 in the

program's menu).

But  look  again.   On  a  Mercator map,  the poles cannot be

shown because they lie at infinity,  and the spacing between par-

allels  increases  with increasing distance from the equator.   A

region  at  the  60th  parallel  plots twice as large as the same

area at the equator.  Moved to 80 degrees, that same area becomes

33 times larger than at the equator!   The Mercator is said to be

conformal  --  it  maintains the shapes of small areas at the ex-

pense of distorting sizes.   In contrast,  the Equidistant Cylin-

drical preserves neither.

Map  2:  The  Equidistant  Cylindrical

o  Preserves neither shapes nor areas;

o  Poles shown as lines;

o  Meridians and parallels are equidistant

straight lines intersecting at right angles;

o  Easy to compute.

Imagine  a  globe  tucked  inside a cylinder so that only the

equator  touches  the  sides.   That's the basic idea behind this

and  all  other  so-called  "cylindrical"  projections.    "Graph

paper"  may  be  the  best way to describe the Equidistant Cylin-

drical.   In  this   projection's   simplest  form the equator is

chosen  to  be  the standard parallel,  called Phi1.   This means

that  the  equator  is  the  latitude displayed true to scale and

without  distortion.   If   meridians   and parallels are equally

spaced,  the result is a grid of tiny squares twice as long as it

is wide.  Experiment with different values of Phi1 and watch what

happens to the map.

All  in  all,  the Equidistant Cylindrical is not a very good

choice for displaying geographic data.  It preserves neither area

nor  shape  and  is best used when distortions are less important

than just getting the data on-screen as quickly as possible.

Map 3: The Sinusoidal

o  Equal area;

o  Central meridian is straight, all others sinusoidal

curves;

o  Parallels equally spaced straight lines;

o  Easy to compute.

The  Sinusoidal  projection  is more useful and only slightly

more  complex.   Developed  in  the 16th century,  the Sinusoidal

shows  areas   correctly   and   exhibits no distortion along the

equator  and central meridian.   Distortion becomes pronounced at

the outer meridians,  extreme near the poles.  Latitude lines are

straight,  parallel,  and equally spaced. The central meridian is

a straight line and crosses all parallels at right angles;  other

meridians  form  curves,  their curvature increasing with greater

distance  from  the  map's  center.   You'll  usually  see    the

Sinusoidal  used for world maps,  often in an "interrupted" form.

In  this  case  the world is broken into sections,  each with its

own  central meridian.   Some atlases also use the Sinusoidal for

South America, Africa, and the central Pacific.

Map 4: The Hammer

o  Equal area;

o  Equator and central meridian are striaght lines;

o  Curvature of meridians increase as distance from

central meridian increases;

o  Curvature of parallels increases with increasing

distance from the Equator.

This  equal-area  projection  was  developed in the late 19th

century  by  a   German   professor of surveying.   It looks more

realistic  than  the  other equal-area projection discussed here,

the  Sinusoidal,  but  this  realism  comes at a  price. Hammer's

projection  has curved parallels and curved meridians,  a feature

which makes it somewhat more difficult to construct.   Of course,

it's a good choice for displaying global distributions because it

shows the whole world at a glance. Climatic data, the movement of

continents,  the distribution of astronomical objects -- all have

been plotted on this projection in recent science journals.

Map 5: The Orthographic

o  Perspective projection from infinite distance (like

the view through a telescope);

o  Only one hemisphere can be shown at a time;

o  Great distortion along the edge of the visible

hemisphere;

o  Azimuthal -- directions from map center to every other

point on the map are shown correctly;

The problem in this projection is to determine whether or not

each point is visible from the viewing location (central meridian

and standard parallel) you choose. The Orthographic presents "the

view  from  infinity,"  the Earth as seen from far away in space.

The result is a map that looks very much like a globe.  It is the

least  useful  map for making measurements because of the extreme

distortion  near  its edges -- in fact,  the center of the map is

the only point without distortion.   Unlike the other projections

mentioned  here,  the Orthographic plots only one hemisphere on a

single map. It is generally reserved for pictorial views, such as

for index maps in an atlas.

Scaling to a Graphics Board

CARTOG.PAS scales these maps for CGA graphics.   To determine

how  you  alter  the graphics constants for your specific system,

you  must  first  examine   the  comment accompanying each of the

mapping procedures.   In Procedure Sinusoidal,  for example,  the

comment states:

For R = 1: -Pi <= X <= Pi  and  -Pi/2 <= Y <= Pi/2.

This  is the key for calculating the proper R to fill a given

graphics screen. R represents the radius of the scaled-down globe

from  which  the map is developed -- it's the radius of the globe

at the scale of the map.  A large-scale map,  such as a road map,

uses  a  very large "model" globe;  a small-scale map,  such as a

world map in an atlas, uses a small generating globe. This number

depends on the map projection, the size of the sheet on which the

map is being drawn, and the region being mapped.

To scale the maps to a computer screen,  you need to know the

values  returned  by  each   map   projection for a globe of unit

radius.  You  also need to know the resolution of your screen and

its aspect ratio -that is, the factor by which you need to divide

the  X  coordinate  to   compensate for non-square pixels.  As an

example, let's find the R for displaying a full Sinusoidal map on

the CGA.  The map's total range is 2Pi in X, Pi in Y. The map's X

axis  must fit within 640 pixels,  the Y axis must fit within 200

pixels. For the CGA, the aspect ratio is 2.4. So for X:

R = {( Screen  - Screen  ) / ( Map -  Map  )} / Aspect

Xmax      Xmin       Xmax   Xmin

= (640 / 2Pi) / 2.4

= 42.4.

And for Y:

R = {(Screen  - Screen)  /  (Map  - Map    )}

Ymax      Ymin        Ymax   Ymin

= 200 / Pi

= 63.6.

The  smallest R controls,  so to fit a full Sinusoidal map on

the  CGA  requires  an   R   of  42.4,  which we rounded to 40 in

CARTOG.PAS.   Note  that  this  is also the correct value for the

Mercator  and Equidistant Cylindrical projections.   As it turned

out,  we  could  use  this  value for all the projections,  so we

defined it as a constant.   In CARTOG,  however, we make each map

fill  the  graphics   screen   by   adjusting R when you choose a

projection.  To make the Hammer and Orthographic maps as large as

possible  on your graphics screen,  you must go through the above

process to find the right R.

If you don't know the aspect ratio of your system,  just plot

the  Orthographic's  grid  and  plug   in values until it forms a

circle.   Depending on your system, you may also need a different

aspect  ratio  to get proper screen dumps from your printer.  The

following  graphics  constants  work  with  most  common  display

resolutions.

EGA:                   Hercules:           VGA:

CONST XCENTER = 320;      XCENTER = 360;      XCENTER = 320;

YCENTER = 174;      YCENTER = 174;      YCENTER = 240;

ASPECT  = 1.37;     ASPECT  = 1.5;      ASPECT  =   1;

R       = 70;       R       = 70;       R       =  70;

The  Data

In  programs  like CARTOG,  which require large quantities of

data,  the file's format should be chosen carefully.  You want to

take  care to keep the size of the file small and facilitate fast

input.   We  decided  against  using an ASCII text file since the

conversion  to binary numbers for the calculations would take too

long;  a  floating-point number format would have required a file

several times larger than the present one.   The non-8087 version

of  Turbo  Pascal uses a floating-point format different from the

8087 version,  further making portability a problem.

We  decided  to  store   the program's longitude and latitude

coordinates  as integers.   We chose to use a Pascal binary file.

The  first  two  bytes of each record is a two-letter ASCII code:

'LS'  means  to draw line strings,  beginning with the data point

in  this record,  and connect  all subsequent points with records

containing  an 'S' code.   (This was done for reasons of downward

compatibility  to  other software and we have not yet bothered to

change  it.   Future  versions  of CARTOG will reduce this to one

byte.)  Each  record   contains   one  data point consisting of a

longitude/latitude   pair.   The   longitudes  and  latitudes are

stored  as   a   (16   bit)  integer representing hundredths of a

degree.   Coordinates  south  of  the   equator and those west of

Greenwich  are  considered  negative.   This  scheme  produces  a

relatively  compact  data  file  giving an adequate resolution of

data points for this purpose.   The WORLD.DAT file distributed by

BYTE  was  compiled  by  the authors from two government-produced

world maps.   For Antarctica,  we used a polar view from the U.S.

Geological  Survey's  1:40,000,000 "World Outline Map";  all else

was  taken  from  the Defense Mapping Agency's "The World," which

is  also available through the USGS.   The resulting binary file,

which consists of about 6,000 latitude/longitude pairs,  provides

a  nice  low-resolution view of the world.

We  have  subsequently   obtained a high-resolution coastline

database  from  the   Goddard   Space Flight Center in Greenbelt,

Maryland.   The  smaller  file  contains  about  15,000    points

(appropriate  to  a  scale of about 1:8,000,000);  the larger one

holds about 95,000 points (1:6,000,000),  including international

boundaries  and  state  boundaries  within  the  U.S.  For a nom-

inal copying/handling charge,  we will be happy to send you these

data.  All data are derived from WORLD DATABANK 1, which was ori-

ginally developed in the 1960s by the U.S. government.

For more information,  feel free to contact me at the address

below:

Francis Reddy

c/o Parnau Graphics, Inc.

2857 S. 160th St.

New Berlin, WI  53151

Or call:

CompuServe (E-MAIL to Jeff Parnau)        : 71555,126

Exec-PC BBS                               : 414-964-5160

TOPAZ BBS (E-MAIL to SYSOP or Frank Reddy): 414-784-6458

- - - - - - E N D - - - - - - -